Arm_learning

#!/usr/bin/env python3

#coding=utf-8

import time

from Arm_Lib import Arm_Device

Arm = Arm_Device()

time.sleep(1)

Arm.Arm_Button_Mode(1)

Arm.Arm_Action_Study()

Arm.Arm_Button_Mode(0)

num = Arm_Read_Action_Num()

print(num)

num = Arm.Read_Action_Num()

print(num)

num = Arm.Arm_Read_Action_Num()

print(num)

Arm.Arm_Action_Mode(1)

Arm.Arm_Action_Mode(2)

Arm.Arm_Action_Mode(0)

Arm.Arm_Clear_Action()

del Arm

build

#!/usr/bin/env python3

#coding=utf-8

import time

from Arm_Lib import Arm_Device

# 获取机械臂对象

Arm = Arm_Device()

time.sleep(.1)

def arm_clamp_block(enable):

if enable == 0:

# enable=0：夹爪张开（舵机6角度60，耗时400ms）

Arm.Arm_serial_servo_write(6, 60, 400)

else:

# enable=1：夹爪夹紧（舵机6角度135，耗时400ms）

Arm.Arm_serial_servo_write(6, 150, 400)

time.sleep(.5)

def arm_move(p, s_time = 500):

for i in range(5):

id = i + 1

if id == 5:

time.sleep(.1)

Arm.Arm_serial_servo_write(id, p[i], int(s_time*1.2))

else :

Arm.Arm_serial_servo_write(id, p[i], s_time)

time.sleep(.01)

time.sleep(s_time/1000)

def arm_move_up():

Arm.Arm_serial_servo_write(2, 90, 1500)

Arm.Arm_serial_servo_write(3, 90, 1500)

Arm.Arm_serial_servo_write(4, 90, 1500)

time.sleep(.1)

p_mould = [90, 130, 0, 0, 90]

p_top = [90, 80, 60, 50, 270]

p_Brown = [90, 53, 30, 36, 270]

p_Yellow = [64, 23, 68, 56, 270]

p_Red = [114, 22, 66, 56, 270]

p_Green = [134, 65, 20, 29, 270]

p_Blue = [44, 65, 20, 28, 270]

p_Brown_yellow = [90, 63, 50, 22, 270]   # 顶层黄色积木

p_Brown_red    = [90, 80, 30, 26, 270]   # 第二层红色积木

p_Brown_green  = [90, 58, 45, 21, 270]   # 第三层绿色积木

p_Brown_blue   = [90, 50, 45, 25, 270]   # 底层蓝色积木

arm_clamp_block(0)

arm_move(p_mould, 1000)

time.sleep(1)

arm_move(p_top, 1000)

arm_move(p_Brown_yellow, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_Yellow, 1000)

arm_clamp_block(0)

time.sleep(1)

arm_move(p_top, 1000)

arm_move(p_Brown_red, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_Red, 1000)

arm_clamp_block(0)

time.sleep(1)

arm_move(p_top, 1000)

arm_move(p_Brown_green, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_Green, 1000)

arm_clamp_block(0)

time.sleep(1)

arm_move(p_top, 1000)

arm_move(p_Brown_blue, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_Blue, 1000)

arm_clamp_block(0)

time.sleep(1)

arm_clamp_block(0)

arm_move(p_mould, 1000)

time.sleep(1)

Clamp_blocks

#!/usr/bin/env python3

#coding=utf-8

import time

from Arm_Lib import Arm_Device

# 获取机械臂对象

Arm = Arm_Device()

time.sleep(.1)

def arm_clamp_block(enable):

if enable == 0:

# enable=0：夹爪张开（舵机6角度60，耗时400ms）

Arm.Arm_serial_servo_write(6, 60, 400)

else:

# enable=1：夹爪夹紧（舵机6角度135，耗时400ms）

Arm.Arm_serial_servo_write(6, 155, 400)

time.sleep(.5)

def arm_move(p, s_time = 500):

for i in range(5):

id = i + 1

if id == 5:

time.sleep(.1)

Arm.Arm_serial_servo_write(id, p[i], int(s_time*1.2))

else :

Arm.Arm_serial_servo_write(id, p[i], s_time)

time.sleep(.01)

time.sleep(s_time/1000)

def arm_move_up():

Arm.Arm_serial_servo_write(2, 90, 1500)

Arm.Arm_serial_servo_write(3, 90, 1500)

Arm.Arm_serial_servo_write(4, 90, 1500)

time.sleep(.1)

p_mould = [90, 130, 0, 0, 90]

p_top = [90, 80, 60, 50, 270]

p_Brown = [90, 53, 30, 36, 270]

p_Yellow = [64, 23, 68, 56, 270]

p_Red = [114, 22, 66, 56, 270]

p_Green = [134, 65, 20, 29, 270]

p_Blue = [44, 65, 20, 28, 270]

arm_clamp_block(0)

arm_move(p_mould, 1000)

time.sleep(1)

arm_move(p_top, 1000)

arm_move(p_Brown, 1000)

arm_clamp_block(1)  # 抓住积木

arm_move(p_top, 1000)

arm_move(p_Yellow, 1000)

arm_clamp_block(0)  # 松开积木

arm_move(p_mould, 1000)

time.sleep(1)

arm_move(p_top, 1000)

arm_move(p_Brown, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_Red, 1000)

arm_clamp_block(0)

arm_move_up()

arm_move(p_mould, 1100)

time.sleep(1)

arm_move(p_top, 1000)

arm_move(p_Brown, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_Green, 1000)

arm_clamp_block(0)

arm_move_up()

arm_move(p_mould, 1100)

time.sleep(1)

arm_move(p_top, 1000)

arm_move(p_Brown, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_Blue, 1000)

arm_clamp_block(0)

arm_move_up()

arm_move(p_mould, 1100)

time.sleep(1)

del Arm

dance

import time

from Arm_Lib import Arm_Device

# 获取机械臂对象

Arm = Arm_Device()

# 定义全局延迟参数

MOVE_DELAY = 0.5

# 舞蹈动作循环

try:

while True:

# 动作1

Arm.Arm_serial_servo_write6(90, 60, 60, 90, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作2

Arm.Arm_serial_servo_write6(90, 120, 120, 90, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作3

Arm.Arm_serial_servo_write6(60, 90, 90, 120, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作4

Arm.Arm_serial_servo_write6(120, 90, 90, 60, 90, 90, 500)

time.sleep(MOVE_DELAY)

except KeyboardInterrupt:

# 复位舵机

Arm.Arm_serial_servo_write6(90, 90, 90, 90, 90, 90, 500)

time.sleep(0.5)

print("舞蹈程序结束，机械臂复位")

import time

from Arm_Lib import Arm_Device

# 获取机械臂对象

Arm = Arm_Device()

# 全局动作延迟

MOVE_DELAY = 0.5

try:

while True:

# 动作 1

Arm.Arm_serial_servo_write6(90, 60, 60, 90, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作 2

Arm.Arm_serial_servo_write6(90, 120, 120, 90, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作 3

Arm.Arm_serial_servo_write6(60, 90, 90, 120, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作 4

Arm.Arm_serial_servo_write6(120, 90, 90, 60, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作 5

Arm.Arm_serial_servo_write6(90, 90, 50, 130, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作 6

Arm.Arm_serial_servo_write6(90, 90, 130, 50, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作 7

Arm.Arm_serial_servo_write6(70, 110, 70, 110, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作 8

Arm.Arm_serial_servo_write6(110, 70, 110, 70, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作 9

Arm.Arm_serial_servo_write6(90, 70, 120, 70, 60, 90, 500)

time.sleep(MOVE_DELAY)

# 动作 10

Arm.Arm_serial_servo_write6(90, 120, 70, 120, 120, 90, 500)

time.sleep(MOVE_DELAY)

except KeyboardInterrupt:

# 按下 Ctrl+C 停止，复位机械臂

Arm.Arm_serial_servo_write6(90, 90, 90, 90, 90, 90, 500)

time.sleep(0.5)

print("舞蹈程序结束，机械臂已复位至中位")

import time

from Arm_Lib import Arm_Device

# 获取机械臂对象

Arm = Arm_Device()

# 全局动作延迟

MOVE_DELAY = 0.5

try:

while True:

# 动作 1

Arm.Arm_serial_servo_write6(90, 60, 60, 90, 90, 150, 500)

time.sleep(MOVE_DELAY)

# 动作 2

Arm.Arm_serial_servo_write6(90, 120, 120, 90, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作 3

Arm.Arm_serial_servo_write6(60, 90, 90, 120, 90, 150, 500)

time.sleep(MOVE_DELAY)

# 动作 4

Arm.Arm_serial_servo_write6(120, 90, 90, 60, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作 5

Arm.Arm_serial_servo_write6(90, 90, 50, 130, 90, 150, 500)

time.sleep(MOVE_DELAY)

# 动作 6

Arm.Arm_serial_servo_write6(90, 90, 130, 50, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作 7

Arm.Arm_serial_servo_write6(70, 110, 70, 110, 90, 150, 500)

time.sleep(MOVE_DELAY)

# 动作 8

Arm.Arm_serial_servo_write6(110, 70, 110, 70, 90, 90, 500)

time.sleep(MOVE_DELAY)

# 动作 9

Arm.Arm_serial_servo_write6(90, 70, 120, 70, 60, 150, 500)

time.sleep(MOVE_DELAY)

# 动作 10

Arm.Arm_serial_servo_write6(90, 120, 70, 120, 120, 90, 500)

time.sleep(MOVE_DELAY)

except KeyboardInterrupt:

# 按下 Ctrl+C 停止，复位机械臂（夹爪也回到中位）

Arm.Arm_serial_servo_write6(90, 90, 90, 90, 90, 90, 500)

time.sleep(0.5)

print("舞蹈程序结束，机械臂已复位至中位")

dancing

import time

from Arm_Lib import Arm_Device

# 初始化机械臂

Arm = Arm_Device()

MOVE_DELAY = 0.9    # 慢速，保证复杂动作走完

MOVE_SPEED = 900    # 舵机运动毫秒时长

try:

while True:

# ====================== 开场三段式鞠躬：左 / 中 / 右 ======================

# 预备站姿

Arm.Arm_serial_servo_write6(90, 90, 90, 90, 90, 90, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 左方位鞠躬

Arm.Arm_serial_servo_write6(55, 112, 68, 85, 55, 70, MOVE_SPEED)

time.sleep(MOVE_DELAY)

Arm.Arm_serial_servo_write6(90, 90, 90, 90, 90, 90, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 正中方位鞠躬

Arm.Arm_serial_servo_write6(90, 116, 63, 90, 45, 65, MOVE_SPEED)

time.sleep(MOVE_DELAY)

Arm.Arm_serial_servo_write6(90, 90, 90, 90, 90, 90, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 右方位鞠躬

Arm.Arm_serial_servo_write6(125, 112, 68, 95, 55, 115, MOVE_SPEED)

time.sleep(MOVE_DELAY)

Arm.Arm_serial_servo_write6(90, 90, 90, 90, 90, 90, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# ====================== 30组复杂联动舞蹈动作 ======================

# 1 左上方圆弧舒展

Arm.Arm_serial_servo_write6(60, 55, 115, 70, 35, 55, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 2 右上方圆弧舒展

Arm.Arm_serial_servo_write6(120, 55, 115, 110, 35, 125, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 3 高位居中舒展

Arm.Arm_serial_servo_write6(90, 50, 120, 90, 30, 90, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 4 左侧波浪下沉

Arm.Arm_serial_servo_write6(50, 95, 75, 75, 60, 65, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 5 右侧波浪下沉

Arm.Arm_serial_servo_write6(130, 95, 75, 105, 60, 115, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 6 中位俯身柔摆

Arm.Arm_serial_servo_write6(90, 110, 68, 90, 50, 90, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 7 手腕左螺旋转动

Arm.Arm_serial_servo_write6(70, 75, 100, 55, 40, 120, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 8 手腕右螺旋转动

Arm.Arm_serial_servo_write6(110, 75, 100, 125, 40, 60, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 9 向前缓慢托举

Arm.Arm_serial_servo_write6(90, 60, 62, 85, 135, 80, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 10 向后缓缓收回

Arm.Arm_serial_servo_write6(90, 105, 98, 95, 35, 100, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 11 左侧低位环绕画圆

Arm.Arm_serial_servo_write6(45, 90, 85, 65, 70, 110, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 12 右侧低位环绕画圆

Arm.Arm_serial_servo_write6(135, 90, 85, 115, 70, 70, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 13 左上斜向伸展

Arm.Arm_serial_servo_write6(65, 60, 110, 60, 45, 50, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 14 右下斜向伸展

Arm.Arm_serial_servo_write6(115, 120, 55, 120, 75, 130, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 15 右上斜向伸展

Arm.Arm_serial_servo_write6(115, 60, 110, 120, 45, 130, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 16 左下斜向伸展

Arm.Arm_serial_servo_write6(65, 120, 55, 60, 75, 50, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 17 连续柔化左波浪

Arm.Arm_serial_servo_write6(70, 80, 95, 70, 50, 105, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 18 连续柔化右波浪

Arm.Arm_serial_servo_write6(110, 80, 95, 110, 50, 75, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 19 缓慢正向抓握

Arm.Arm_serial_servo_write6(90, 70, 65, 90, 140, 95, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 20 缓慢反向张开

Arm.Arm_serial_servo_write6(90, 100, 92, 90, 35, 85, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 21 高位左环绕

Arm.Arm_serial_servo_write6(55, 50, 122, 75, 40, 60, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 22 高位右环绕

Arm.Arm_serial_servo_write6(125, 50, 122, 105, 40, 120, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 23 中心小圆弧旋转

Arm.Arm_serial_servo_write6(80, 85, 90, 80, 55, 100, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 24 反向中心小圆弧旋转

Arm.Arm_serial_servo_write6(100, 85, 90, 100, 55, 80, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 25 低位整体柔摆

Arm.Arm_serial_servo_write6(75, 110, 60, 85, 65, 110, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 26 反向低位整体柔摆

Arm.Arm_serial_servo_write6(105, 110, 60, 95, 65, 70, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 27 中位优雅抬臂

Arm.Arm_serial_servo_write6(90, 65, 105, 90, 40, 90, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 28 中位优雅落臂

Arm.Arm_serial_servo_write6(90, 108, 70, 90, 60, 90, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 29 居中柔和预备姿态

Arm.Arm_serial_servo_write6(90, 90, 90, 90, 50, 90, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# 30 最终优雅定格展臂

Arm.Arm_serial_servo_write6(90, 72, 98, 90, 35, 90, MOVE_SPEED)

time.sleep(MOVE_DELAY)

# ====================== 结尾正式鞠躬收势 ======================

Arm.Arm_serial_servo_write6(90, 120, 60, 90, 45, 90, MOVE_SPEED)

time.sleep(MOVE_DELAY)

Arm.Arm_serial_servo_write6(90, 90, 90, 90, 90, 90, MOVE_SPEED)

time.sleep(MOVE_DELAY)

except KeyboardInterrupt:

Arm.Arm_serial_servo_write6(90, 90, 90, 90, 90, 90, 900)

time.sleep(0.9)

print("舞蹈结束，机械臂复位完成")

Face

import cv2 as cv

import os

import threading

import ipywidgets as widgets

from IPython.display import display

from Arm_Lib import Arm_Device

import Arm_Lib

Arm = Arm_Device()

joints_0 = [90, 135, 20, 25, 90, 30]

Arm.Arm_serial_servo_write6_array(joints_0, 1000)

class PositionalPID:

def __init__(self, P, I, D):

self.Kp = P

self.Ki = I

self.Kd = D

self.SystemOutput = 0.0

self.ResultValueBack = 0.0

self.PidOutput = 0.0

self.PIDErrADD = 0.0

self.ErrBack = 0.0

def SetStepSignal(self, StepSignal):

Err = StepSignal - self.SystemOutput

KpWork = self.Kp * Err

KiWork = self.Ki * self.PIDErrADD

KdWork = self.Kd * (Err - self.ErrBack)

self.PidOutput = KpWork + KiWork + KdWork

self.PIDErrADD += Err

if self.PIDErrADD > 500:

self.PIDErrADD = 500

if self.PIDErrADD < -500:

self.PIDErrADD = -500

self.ErrBack = Err

def SetInertiaTime(self, InertiaTime, SampleTime):

self.SystemOutput = (InertiaTime * self.ResultValueBack +

SampleTime * self.PidOutput) / (SampleTime + InertiaTime)

self.ResultValueBack = self.SystemOutput

class face_follow:

# 模型 URL（类级常量，不依赖外部变量）

_PROTOTXT_URL = ("https://raw.githubusercontent.com/opencv/opencv/"

"master/samples/dnn/face_detector/deploy.prototxt")

_CAFFEMODEL_URL = ("https://github.com/opencv/opencv_3rdparty/raw/"

"dnn_samples_face_detector_20170830/"

"res10_300x300_ssd_iter_140000.caffemodel")

@staticmethod

def _get_model_dir():

"""获取模型文件所在目录（Jupyter 兼容，使用当前工作目录）"""

import os

return os.getcwd()

@classmethod

def _ensure_model(cls, prototxt_path, caffemodel_path):

"""自动下载 DNN 模型文件，网络不通则提示手动下载"""

import os

import urllib.request

missing = []

for path, url, name in [

(prototxt_path, cls._PROTOTXT_URL, "deploy.prototxt"),

(caffemodel_path, cls._CAFFEMODEL_URL,

"res10_300x300_ssd_iter_140000.caffemodel"),

]:

if os.path.exists(path):

continue

print("下载 %s ..." % name)

try:

urllib.request.urlretrieve(url, path)

print("%s 下载完成" % name)

except Exception as e:

print("下载 %s 失败: %s" % (name, e))

missing.append((name, url))

if missing:

print("\n" + "=" * 55)

print("以下模型文件缺失，请手动下载后放到程序目录：")

for name, url in missing:

print("  %s\n    %s" % (name, url))

print("=" * 55)

def __init__(self):

import os

self.img = None

self.target_servox = 90

self.target_servoy = 45

self.Arm = Arm_Lib.Arm_Device()

# ===== 模型路径 =====

model_dir = self._get_model_dir()

prototxt = os.path.join(model_dir, "deploy.prototxt")

caffemodel = os.path.join(model_dir, "res10_300x300_ssd_iter_140000.caffemodel")

# ===== 检测模式选择 =====

self.detect_mode = None   # "dnn" 或 "haar"

if os.path.exists(prototxt) and os.path.exists(caffemodel):

# DNN 模式

self.net = cv.dnn.readNetFromCaffe(prototxt, caffemodel)

try:

self.net.setPreferableBackend(cv.dnn.DNN_BACKEND_OPENCV)

self.net.setPreferableTarget(cv.dnn.DNN_TARGET_OPENCL)

except Exception:

pass

self.detect_mode = "dnn"

print("DNN 人脸检测模型加载成功")

else:

# Haar Cascade 回退：使用当前目录下的本地文件

cascade_path = os.path.join(os.getcwd(), "haarcascade_frontalface_default.xml")

self.face_cascade = cv.CascadeClassifier(cascade_path)

if self.face_cascade.empty():

raise RuntimeError("无法加载 Haar Cascade: %s" % cascade_path)

self.detect_mode = "haar"

print("Haar Cascade 人脸检测就绪: %s" % cascade_path)

# ===== 检测置信度阈值 =====

self.confidence_threshold = 0.6      # DNN 正常阈值

self.low_confidence_threshold = 0.35 # DNN 低阈值回退

# ===== 边缘检测参数 =====

self.edge_margin = 15

self.recovery_gain_x = 50

self.recovery_gain_y = 40

# ===== PID 参数 =====

self.xservo_pid = PositionalPID(0.8, 0.08, 0.2)

self.yservo_pid = PositionalPID(0.8, 0.08, 0.2)

# ===== 低通滤波 =====

self.smooth_x = 160.0

self.smooth_y = 120.0

self.alpha = 0.4

# ===== 死区 / 限幅 =====

self.dead_zone = 15

self.max_angle_change = 6

self.recovery_max_angle_change = 12

self.last_target_servox = 90

self.last_target_servoy = 45

self.face_lost_count = 0

self.max_lost_frames = 15

self.last_face_pos = None

self.face_detected = False

# 帧跳过

self.frame_count = 0

self.detect_interval = 3

self._cached_result = None

def _extract_faces_dnn(self, detections, h, w, min_conf, min_size):

"""从 DNN 检测结果中提取人脸框"""

faces = []

for i in range(detections.shape[2]):

confidence = detections[0, 0, i, 2]

if confidence < min_conf:

continue

box = detections[0, 0, i, 3:7] * [w, h, w, h]

(x1, y1, x2, y2) = box.astype("int")

x1, y1 = max(0, x1), max(0, y1)

x2, y2 = min(w, x2), min(h, y2)

fw, fh = x2 - x1, y2 - y1

if fw > min_size and fh > min_size:

faces.append((x1, y1, fw, fh, confidence))

return faces

def _detect_face_dnn(self, bgr):

"""DNN SSD 人脸检测"""

h, w = bgr.shape[:2]

blob = cv.dnn.blobFromImage(bgr, 1.0, (300, 300),

(104.0, 177.0, 123.0))

self.net.setInput(blob)

detections = self.net.forward()

faces = self._extract_faces_dnn(detections, h, w,

self.confidence_threshold, 15)

secondary_detect = (len(faces) == 0)

if secondary_detect:

faces = self._extract_faces_dnn(detections, h, w,

self.low_confidence_threshold, 10)

return faces, secondary_detect

def _detect_face_haar(self, gray):

"""Haar Cascade 人脸检测（多轮降级策略）"""

faces_raw = self.face_cascade.detectMultiScale(

gray, scaleFactor=1.05, minNeighbors=5,

minSize=(30, 30), flags=cv.CASCADE_SCALE_IMAGE)

if len(faces_raw) == 0:

# 降级：更宽松的参数

faces_raw = self.face_cascade.detectMultiScale(

gray, scaleFactor=1.03, minNeighbors=3,

minSize=(20, 20), flags=cv.CASCADE_SCALE_IMAGE)

faces = []

for (x, y, w, h) in faces_raw:

confidence = min(w * h / 20000.0, 0.95)  # 估算置信度

faces.append((x, y, w, h, confidence))

return faces, False

def _check_edge_status(self, fx, fy, fw, fh, w, h):

"""检测人脸是否贴近画面边缘（不完整人脸）"""

return {

'left':   fx <= self.edge_margin,

'right':  (fx + fw) >= (w - self.edge_margin),

'top':    fy <= self.edge_margin,

'bottom': (fy + fh) >= (h - self.edge_margin),

'any':    False  # 占位，后续更新

}

def detect_face(self, bgr):

"""

人脸检测（DNN 或 Haar Cascade，取决于 detect_mode）

返回 (center_x, center_y, w, h, faces, edge_status) 或 None

"""

h, w = bgr.shape[:2]

if self.detect_mode == "dnn":

faces, secondary_detect = self._detect_face_dnn(bgr)

else:

gray = cv.cvtColor(bgr, cv.COLOR_BGR2GRAY)

faces, secondary_detect = self._detect_face_haar(gray)

if len(faces) == 0:

self.last_face_pos = None

return None

# 按 面积*置信度 排序

faces.sort(key=lambda f: f[2] * f[3] * f[4], reverse=True)

# 时序优先：靠近上一帧位置的加权

if self.last_face_pos is not None and len(faces) > 1:

lx, ly, lw, lh = self.last_face_pos

lcx, lcy = lx + lw / 2, ly + lh / 2

def score(f):

fx, fy, fw_, fh_, conf = f

fcx, fcy = fx + fw_ / 2, fy + fh_ / 2

dist = ((fcx - lcx) ** 2 + (fcy - lcy) ** 2) ** 0.5 + 1

return fw_ * fh_ * conf / dist

faces.sort(key=score, reverse=True)

(fx, fy, fw, fh, conf) = faces[0]

center_x = fx + fw / 2

center_y = fy + fh / 2

self.last_face_pos = (fx, fy, fw, fh)

# 检测人脸是否不完整（贴边）

edge_status = self._check_edge_status(fx, fy, fw, fh, w, h)

edge_status['any'] = any([edge_status['left'], edge_status['right'],

edge_status['top'], edge_status['bottom']])

edge_status['secondary'] = secondary_detect

# 返回不含置信度的框

faces_clean = [(fx, fy, fw, fh) for (fx, fy, fw, fh, _) in faces]

return center_x, center_y, fw, fh, faces_clean, edge_status

def follow_function(self, img):

self.img = img

h, w = self.img.shape[:2]

cx, cy = w // 2, h // 2  # 画面中心

self.frame_count += 1

if self.frame_count % self.detect_interval == 0:

result = self.detect_face(self.img)

self._cached_result = result

else:

result = self._cached_result

if result is not None:

face_x, face_y, face_w, face_h, faces, edge_status = result

self.face_detected = True

self.face_lost_count = 0

# ===== 绘制人脸框 =====

for (fx, fy, fw, fh) in faces:

# 不完整人脸用橙色框，正常用绿色框

color = (0, 165, 255) if edge_status['any'] else (0, 255, 0)

thickness = 3 if edge_status['any'] else 2

cv.rectangle(self.img, (fx, fy), (fx + fw, fy + fh), color, thickness)

cv.circle(self.img, (int(fx + fw / 2), int(fy + fh / 2)),

5, (0, 0, 255), -1)

# ===== 计算边缘恢复补偿 =====

recovery_x = 0.0

recovery_y = 0.0

if edge_status['left']:

recovery_x = self.recovery_gain_x   # 左贴边 → 目标偏右 → 摄像头左转

elif edge_status['right']:

recovery_x = -self.recovery_gain_x  # 右贴边 → 目标偏左 → 摄像头右转

if edge_status['top']:

recovery_y = self.recovery_gain_y   # 上贴边 → 目标偏下 → 摄像头上仰

elif edge_status['bottom']:

recovery_y = -self.recovery_gain_y  # 下贴边 → 目标偏上 → 摄像头下俯

# PID 目标点 = 画面中心 + 恢复补偿偏移

pid_target_x = cx + recovery_x

pid_target_y = cy + recovery_y

# 绘制恢复方向箭头

if edge_status['any']:

# 画面中心十字准星

cv.line(self.img, (cx - 15, cy), (cx + 15, cy), (0, 255, 255), 1)

cv.line(self.img, (cx, cy - 15), (cx, cy + 15), (0, 255, 255), 1)

# 从人脸中心指向恢复目标

pt1 = (int(face_x), int(face_y))

pt2 = (int(pid_target_x), int(pid_target_y))

cv.arrowedLine(self.img, pt1, pt2, (0, 255, 255), 2, tipLength=0.3)

# 文字提示

cv.putText(self.img, "Partial Face - Recovering", (5, 50),

cv.FONT_HERSHEY_SIMPLEX, 0.5, (0, 165, 255), 2)

self.smooth_x = self.alpha * face_x + (1 - self.alpha) * self.smooth_x

self.smooth_y = self.alpha * face_y + (1 - self.alpha) * self.smooth_y

status_text = "Face Locked" if not edge_status['any'] else "Partial Face"

if self.detect_mode == "haar":

status_text += " [Haar]"

status_color = (0, 255, 0) if not edge_status['any'] else (0, 165, 255)

cv.putText(self.img, status_text, (5, 20),

cv.FONT_HERSHEY_SIMPLEX, 0.6, status_color, 2)

# 边缘检测二级命中提示

if edge_status.get('secondary'):

cv.putText(self.img, "Lo-Conf Detect", (5, 75),

cv.FONT_HERSHEY_SIMPLEX, 0.45, (0, 200, 255), 1)

# ===== X 轴 PID（含恢复偏移） =====

x_error = abs(self.smooth_x - pid_target_x)

if x_error > self.dead_zone:

self.xservo_pid.SystemOutput = self.smooth_x

self.xservo_pid.SetStepSignal(pid_target_x)

self.xservo_pid.SetInertiaTime(0.01, 0.1)

target_valuex = int(1500 + self.xservo_pid.SystemOutput)

target_servox_new = int((target_valuex - 500) / 10)

delta_x = target_servox_new - self.last_target_servox

# 不完整人脸时允许更大步长以便快速恢复

max_step = self.recovery_max_angle_change if edge_status['any'] else self.max_angle_change

if abs(delta_x) > max_step:

delta_x = max_step if delta_x > 0 else -max_step

self.target_servox = self.last_target_servox + delta_x

self.target_servox = max(0, min(180, self.target_servox))

# ===== Y 轴 PID（含恢复偏移） =====

y_error = abs(self.smooth_y - pid_target_y)

if y_error > self.dead_zone:

self.yservo_pid.SystemOutput = self.smooth_y

self.yservo_pid.SetStepSignal(pid_target_y)

self.yservo_pid.SetInertiaTime(0.01, 0.1)

target_valuey = int(1500 + self.yservo_pid.SystemOutput)

target_servoy_new = int((target_valuey - 500) / 10) - 45

delta_y = target_servoy_new - self.last_target_servoy

max_step = self.recovery_max_angle_change if edge_status['any'] else self.max_angle_change

if abs(delta_y) > max_step:

delta_y = max_step if delta_y > 0 else -max_step

self.target_servoy = self.last_target_servoy + delta_y

self.target_servoy = max(0, min(360, self.target_servoy))

self.last_target_servox = self.target_servox

self.last_target_servoy = self.target_servoy

joints_0 = [self.target_servox, 135, self.target_servoy / 2,

self.target_servoy / 2, 90, 30]

self.Arm.Arm_serial_servo_write6_array(joints_0, 1000)

else:

self.face_lost_count += 1

self.face_detected = False

cv.putText(self.img, "Face Lost: %d" % self.face_lost_count, (5, 20),

cv.FONT_HERSHEY_SIMPLEX, 0.6, (0, 0, 255), 2)

# ===== 绘制画面边缘警戒线 =====

m = self.edge_margin

cv.rectangle(self.img, (m, m), (w - m, h - m), (80, 80, 80), 1)

return self.img

face_tracker = None   # 懒加载，点击 Face Follow 时才初始化

model = 'General'

# ===== UI =====

button_layout = widgets.Layout(width='200px', height='80px', align_self='center')

face_follow_btn = widgets.Button(description='Face Follow', button_style='success',

layout=button_layout)

follow_cancel = widgets.Button(description='Stop Follow', button_style='danger',

layout=button_layout)

exit_button = widgets.Button(description='Exit', button_style='danger',

layout=button_layout)

imgbox = widgets.Image(format='jpg', height=320, width=480,

layout=widgets.Layout(align_self='auto'))

img_box = widgets.VBox([imgbox], layout=widgets.Layout(align_self='auto'))

Slider_box = widgets.VBox([face_follow_btn, follow_cancel, exit_button],

layout=widgets.Layout(align_self='auto'))

controls_box = widgets.HBox([img_box, Slider_box],

layout=widgets.Layout(align_self='auto'))

def face_follow_Callback(value):

global model, face_tracker

if face_tracker is None:

try:

face_tracker = face_follow()

except Exception as e:

print("人脸追踪初始化失败: %s" % e)

face_tracker = None

return

model = 'face_follow'

print("人脸追踪已启动 (DNN SSD)")

def follow_cancel_Callback(value):

global model

model = 'General'

print("追踪已取消")

def exit_button_Callback(value):

global model

model = 'Exit'

print("程序退出")

face_follow_btn.on_click(face_follow_Callback)

follow_cancel.on_click(follow_cancel_Callback)

exit_button.on_click(exit_button_Callback)

def camera():

global model

capture = cv.VideoCapture(0)

capture.set(3, 320)

capture.set(4, 240)

capture.set(5, 30)

while capture.isOpened():

try:

_, img = capture.read()

img = cv.resize(img, (320, 240))

if model == 'face_follow' and face_tracker is not None:

img = face_tracker.follow_function(img)

if model == 'Exit':

cv.destroyAllWindows()

capture.release()

break

imgbox.value = cv.imencode('.jpg', img,

[cv.IMWRITE_JPEG_QUALITY, 50])[1].tobytes()

except KeyboardInterrupt:

capture.release()

devices = []

for i in range(5):

cap = cv.VideoCapture(i, cv.CAP_DSHOW)

if cap.isOpened():

devices.append(i)

cap.release()

print("可用设备索引:", devices)

display(controls_box)

camera_thread = threading.Thread(target=camera, daemon=True)

camera_thread.start()

face2

import cv2

import numpy as np

import ipywidgets as widgets

import threading

import time

from IPython.display import display, clear_output

# 全局状态变量

model = "General"

exit_flag = False

servo_x_angle = 90

servo_y_angle = 90

# ========== 修复后的PID（重点：增加误差重置、更强参数） ==========

class PosPID:

def __init__(self, kp, ki, kd, output_min, output_max):

self.kp = kp

self.ki = ki

self.kd = kd

self.output_min = output_min

self.output_max = output_max

self.integral = 0

self.last_error = 0

def calculate(self, setpoint, feedback):

# 【修复】你之前写反了！！！！

error = feedback - setpoint

p = self.kp * error

self.integral += error

i = self.ki * self.integral

d = self.kd * (error - self.last_error)

# 防止积分累积卡死

if abs(error) < 5:

self.integral = 0

self.last_error = error

output = p + i + d

output = np.clip(output, self.output_min, self.output_max)

return output

# 【修复】强化追踪参数，能肉眼看到追踪

pid_x = PosPID(kp=0.4, ki=0.02, kd=0.1, output_min=-8, output_max=8)

pid_y = PosPID(kp=0.4, ki=0.02, kd=0.1, output_min=-8, output_max=8)

class face_follow:

def __init__(self):

# 【终极解决路径问题】使用opencv内置模型，不用上传文件

self.face_cascade = cv2.CascadeClassifier(cv2.data.haarcascades

+ "haarcascade_frontalface_default.xml")

self.target_x = 320

self.target_y = 240

def face_filter(self, faces):

max_face = None

max_area = 0

for (x, y, w, h) in faces:

if w < 20 or h < 20:   # 过滤太小人脸

continue

area = w * h

if area > max_area:

max_area = area

max_face = (x, y, w, h)

return max_face

def follow_function(self, frame):

global servo_x_angle, servo_y_angle

frame = cv2.resize(frame, (640, 480))

gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

faces = self.face_cascade.detectMultiScale(

gray,

scaleFactor=1.2,

minNeighbors=4,

minSize=(20, 20)

)

target_face = self.face_filter(faces)

if target_face is not None:

x, y, w, h = target_face

cv2.rectangle(frame, (x, y), (x+w, y+h), (0, 255, 0), 2)

face_cx = x + w // 2

face_cy = y + h // 2

cv2.circle(frame, (face_cx, face_cy), 4, (0, 0, 255), -1)

# 【核心修复】删除你原来错误的判断条件，人脸在画面里就持续追踪

delta_x = pid_x.calculate(self.target_x, face_cx)

servo_x_angle += delta_x

servo_x_angle = np.clip(servo_x_angle, 0, 180)

delta_y = pid_y.calculate(self.target_y, face_cy)

servo_y_angle += delta_y

servo_y_angle = np.clip(servo_y_angle, 0, 180)

# 打印角度变化，方便你观察是否追踪

print(f"X角度:{servo_x_angle:.1f}  Y角度:{servo_y_angle:.1f}")

return frame

face_tracker = face_follow()

# 图像显示控件

img_widget = widgets.Image(format='jpeg', width=640, height=480)

# 退出按钮（补全 Layout 的闭合括号！）

exit_button = widgets.Button(

description="Exit",

button_style='danger',

layout=widgets.Layout(width='150px')

)

def exit_button_Callback(b):

global exit_flag, model

model = "Exit"

exit_flag = True

exit_button.on_click(exit_button_Callback)

# 布局

ui = widgets.VBox([

img_widget,

widgets.HBox([exit_button], layout=widgets.Layout(justify_content='center'))

])

display(ui)

def main_loop():

global exit_flag

cap = cv2.VideoCapture(0)

cap.set(cv2.CAP_PROP_FRAME_WIDTH, 640)

cap.set(cv2.CAP_PROP_FRAME_HEIGHT, 480)

while not exit_flag:

ret, frame = cap.read()

if not ret:

break

frame = face_tracker.follow_function(frame)

# 编码图像用于jupyter显示

_, jpg_buf = cv2.imencode('.jpg', frame, [cv2.IMWRITE_JPEG_QUALITY, 80])

img_widget.value = jpg_buf.tobytes()

time.sleep(0.03)

cap.release()

cv2.destroyAllWindows()

clear_output()

# 启动追踪线程

track_thread = threading.Thread(target=main_loop, daemon=True)

track_thread.start()

X角度:98.0  Y角度:82.0

X角度:106.0  Y角度:74.0

X角度:114.0  Y角度:66.0

X角度:122.0  Y角度:58.0

X角度:130.0  Y角度:50.0

X角度:138.0  Y角度:42.0

X角度:146.0  Y角度:34.0

X角度:154.0  Y角度:26.0

X角度:162.0  Y角度:18.0

X角度:170.0  Y角度:10.0

X角度:178.0  Y角度:2.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

X角度:180.0  Y角度:0.0

Read_pos

#!/usr/bin/env python3

#coding=utf-8

import time

from Arm_Lib import Arm_Device

Arm = Arm_Device()

time.sleep(1)

def main():

for i in range(6):

aa = Arm.Arm_serial_servo_read(i+1)

print(aa)

time.sleep(0.1)

time.sleep(0.5)

print(" END OF LINE! ")

try:

main()

except KeyboardInterrupt:

print(" Program closed! ")

pass

#!/usr/bin/env python3

#coding=utf-8

import time

from Arm_Lib import Arm_Device

Arm = Arm_Device()

time.sleep(1)

def main():

for i in range(6):

aa = Arm.Arm_serial_servo_read(i+1)

print(aa)

time.sleep(0.1)

time.sleep(0.5)

print(" END OF LINE! ")

try:

main()

except KeyboardInterrupt:

print(" Program closed! ")

pass

#!/usr/bin/env python3

#coding=utf-8

import time

from Arm_Lib import Arm_Device

Arm = Arm_Device()

time.sleep(1)

def main():

for i in range(6):

aa = Arm.Arm_serial_servo_read(i+1)

print(aa)

time.sleep(0.1)

time.sleep(0.5)

print(" END OF LINE! ")

try:

main()

except KeyboardInterrupt:

print(" Program closed! ")

pass

# 控制单个舵机运动，假设控制 ID 为 1 的舵机转动到 90 度

Arm.Arm_serial_servo_write(1, 90, 500)

time.sleep(1)

# 读取该舵机角度

angle = Arm.Arm_serial_servo_read(1)

print(angle)

id = 6

angle = 150

Arm.Arm_serial_servo_write(id, angle, 500)

time.sleep(1)

aa = Arm.Arm_serial_servo_read(id)

print(aa)

time.sleep(.5)

id = 6

angle = 50

Arm.Arm_serial_servo_write(id, angle, 500)

time.sleep(1)

aa = Arm.Arm_serial_servo_read(id)

print(aa)

time.sleep(0.5)

id = 6

angle = 120

Arm.Arm_serial_servo_write(id, angle, 500)

time.sleep(1)

aa = Arm.Arm_serial_servo_read(id)

print(aa)

time.sleep(0.5)

id = 6

angle = 150

Arm.Arm_serial_servo_write(id, angle, 500)

time.sleep(1)

aa = Arm.Arm_serial_servo_read(id)

print(aa)

time.sleep(0.5)

RGB

#!/usr/bin/env python3

#coding-utf-8

import time

from Arm_Lib import Arm_Device

Arm = Arm_Device()

time.sleep(.1)

def main():

while True:

#RGB亮红灯

Arm. Arm_RGB_set (50, 0, 0)

time.sleep (.5)

Arm. Arm_RGB_set (0, 50, 0)

time.sleep(.5)

Arm. Arm_RGB_set (0, 0, 50)

time.sleep(.5)

#RGB亮绿灯

#RGB亮蓝灯

print (" END OF LINE!")

try:

main()

except KeyboardInterrupt:

#释放Arm对象

del Arm

print (" Program closed! ")

pass

up and down

import time

from Arm_Lib import Arm_Device

Arm = Arm_Device()

def ctrl_all_servo(S1, S2, S3, S4, S5, S6, time):

# 假设这个函数在Arm_Lib模块里，需确保模块中有此定义

Arm.Arm_serial_servo_write6(S1, S2, S3, S4, S5, S6, time)

S1 = 90

S2 = 90

S3 = 90

S4 = 90

S5 = 90

S6 = 90

while True:

ctrl_all_servo(S1, S2, S3, S4, S5, S6, 500)

S1 += 10

if S1 > 180:

S1 = 0

# 其他角度类似调整

import time

from Arm_Lib import Arm_Device

Arm = Arm_Device()

def ctrl_all_servo(S1, S2, S3, S4, S5, S6, time):

# 假设这个函数在Arm_Lib模块里，需确保模块中有此定义

Arm.Arm_serial_servo_write6(S1, S2, S3, S4, S5, S6, time)

S1 = 90

S2 = 90

S3 = 90

S4 = 90

S5 = 90

S6 = 90

while True:

ctrl_all_servo(S1, S2, S3, S4, S5, S6, 500)

S4 += 10

if S4 > 135:

S4 = 45

# 其他角度类似调整

import time

from Arm_Lib import Arm_Device

Arm = Arm_Device()

def ctrl_all_servo(S1, S2, S3, S4, S5, S6, move_time):

# 调用六轴舵机同步运动函数

Arm.Arm_serial_servo_write6(S1, S2, S3, S4, S5, S6, move_time)

# 初始角度

S1 = 90

S2 = 90

S3 = 90

S4 = 90

S5 = 90

S6 = 90

try:

while True:

ctrl_all_servo(S1, S2, S3, S4, S5, S6, 500)

S4 += 10

if S4 > 135:

S4 = 45

# 匹配舵机运动时长，增加延时防止指令过快

time.sleep(0.5)

except KeyboardInterrupt:

# 按 Ctrl+C 正常退出，复位舵机

ctrl_all_servo(90, 90, 90, 90, 90, 90, 500)

time.sleep(0.5)

print("程序已停止，舵机复位完成")

Color_Follow文件夹：

111

import cv2 as cv

import threading

import random

from time import sleep

import ipywidgets as widgets

from IPython.display import display

from Arm_Lib import Arm_Device

import Arm_Lib

Arm = Arm_Device()

joints_0 = [90, 135, 20, 25, 90, 30]

Arm.Arm_serial_servo_write6_array(joints_0, 1000)

class PositionalPID:

def __init__(self, P, I, D):

self.Kp = P

self.Ki = I

self.Kd = D

self.SystemOutput = 0.0

self.ResultValueBack = 0.0

self.PidOutput = 0.0

self.PIDErrADD = 0.0

self.ErrBack = 0.0

def SetStepSignal(self, StepSignal):

Err = StepSignal - self.SystemOutput

KpWork = self.Kp * Err

KiWork = self.Ki * self.PIDErrADD

KdWork = self.Kd * (Err - self.ErrBack)

self.PidOutput = KpWork + KiWork + KdWork

self.PIDErrADD += Err

# 积分限幅，防止积分饱和

if self.PIDErrADD > 500:

self.PIDErrADD = 500

if self.PIDErrADD < -500:

self.PIDErrADD = -500

self.ErrBack = Err

def SetInertiaTime(self, InertiaTime, SampleTime):

self.SystemOutput = (InertiaTime * self.ResultValueBack +

SampleTime * self.PidOutput) / (SampleTime + InertiaTime)

self.ResultValueBack = self.SystemOutput

class color_follow:

def __init__(self):

self.img = None

self.target_servox = 90

self.target_servoy = 45

self.Arm = Arm_Lib.Arm_Device()

# ===== 优化1: PID参数（进一步提高灵敏度） =====

self.xservo_pid = PositionalPID(1.5, 0.18, 0.3)

self.yservo_pid = PositionalPID(1.5, 0.18, 0.3)

# ===== 优化2: 低通滤波器，加快响应 =====

self.smooth_x = 320.0

self.smooth_y = 240.0

self.alpha = 0.75          # 提高平滑系数，追踪更跟手

# ===== 优化3: 死区阈值 =====

self.dead_zone = 4         # 缩小死区，微小移动即可响应

# ===== 优化4: 舵机角度变化限幅 =====

self.max_angle_change = 15  # 每帧最大角度变化量

# ===== 优化5: 上一帧角度记录（用于限幅） =====

self.last_target_servox = 90

self.last_target_servoy = 45

def follow_function(self, img, HSV_config):

self.img = cv.resize(img, (640, 480))

self.img = cv.GaussianBlur(self.img, (5, 5), 0)

hsv = cv.cvtColor(self.img, cv.COLOR_BGR2HSV)

# 支持单一HSV范围 或 多个HSV范围（如红色需要双范围）

if isinstance(HSV_config, list):

# 多个HSV范围，合并所有mask

mask = None

for (c_lower, c_upper) in HSV_config:

m = cv.inRange(hsv, c_lower, c_upper)

mask = m if mask is None else cv.bitwise_or(mask, m)

else:

(color_lower, color_upper) = HSV_config

mask = cv.inRange(hsv, color_lower, color_upper)

# 减少形态学操作次数，保留更多细节

mask = cv.erode(mask, None, iterations=1)

mask = cv.dilate(mask, None, iterations=1)

mask = cv.GaussianBlur(mask, (3, 3), 0)

cnts = cv.findContours(mask.copy(), cv.RETR_EXTERNAL, cv.CHAIN_APPROX_SIMPLE)[-2]

if len(cnts) > 0:

cnt = max(cnts, key=cv.contourArea)

(color_x, color_y), color_radius = cv.minEnclosingCircle(cnt)

# 降低最小半径阈值，提高检测灵敏度

if color_radius > 10:

cv.circle(self.img, (int(color_x), int(color_y)), int(color_radius), (255, 0, 255), 2)

# ===== 平滑滤波：防止检测位置突变 =====

self.smooth_x = self.alpha * color_x + (1 - self.alpha) * self.smooth_x

self.smooth_y = self.alpha * color_y + (1 - self.alpha) * self.smooth_y

# ===== X轴PID控制（带动死区判断） =====

x_error = abs(self.smooth_x - 320)

if x_error > self.dead_zone:

self.xservo_pid.SystemOutput = self.smooth_x

self.xservo_pid.SetStepSignal(320)

self.xservo_pid.SetInertiaTime(0.01, 0.1)

target_valuex = int(1500 + self.xservo_pid.SystemOutput)

target_servox_new = int((target_valuex - 500) / 10)

# ===== 角度变化限幅 =====

delta_x = target_servox_new - self.last_target_servox

if abs(delta_x) > self.max_angle_change:

delta_x = self.max_angle_change if delta_x > 0 else -self.max_angle_change

self.target_servox = self.last_target_servox + delta_x

# 边界限制

if self.target_servox > 180:

self.target_servox = 180

if self.target_servox < 0:

self.target_servox = 0

# ===== Y轴PID控制（带动死区判断） =====

y_error = abs(self.smooth_y - 240)

if y_error > self.dead_zone:

self.yservo_pid.SystemOutput = self.smooth_y

self.yservo_pid.SetStepSignal(240)

self.yservo_pid.SetInertiaTime(0.01, 0.1)

target_valuey = int(1500 + self.yservo_pid.SystemOutput)

target_servoy_new = int((target_valuey - 500) / 10) - 45

# ===== 角度变化限幅 =====

delta_y = target_servoy_new - self.last_target_servoy

if abs(delta_y) > self.max_angle_change:

delta_y = self.max_angle_change if delta_y > 0 else -self.max_angle_change

self.target_servoy = self.last_target_servoy + delta_y

if self.target_servoy > 360:

self.target_servoy = 360

if self.target_servoy < 0:

self.target_servoy = 0

# ===== 记录本帧最终角度 =====

self.last_target_servox = self.target_servox

self.last_target_servoy = self.target_servoy

# 调整机械臂各个舵机的旋转角度

joints_0 = [self.target_servox, 135, self.target_servoy / 2, self.target_servoy / 2, 90, 30]

self.Arm.Arm_serial_servo_write6_array(joints_0, 1000)

return self.img

follow = color_follow()

# 初始化模式

model = 'General'

# 初始化HSV_learning值

HSV_learning = ()

# 设置HSV值

# 红色在HSV中跨越0度两端，需要双范围合并检测

color_hsv = {"red": [((0, 50, 50), (10, 255, 255)),      # 红色低端

((156, 50, 50), (180, 255, 255))],    # 红色高端

"green": ((45, 40, 40), (85, 255, 255)),      # 加宽绿色范围

"blue": ((95, 50, 50), (130, 255, 255)),      # 加宽蓝色范围(原110-120)

"yellow": ((22, 40, 50), (45, 255, 255))}     # 微调黄色

# 设置随机颜色

color = [[random.randint(0, 255) for _ in range(3)] for _ in range(255)]

button_layout = widgets.Layout(width='200px', height='100px', align_self='center')

# 输出控件

output = widgets.Output()

# 颜色追踪

color_follow_btn = widgets.Button(description='color_follow', button_style='success', layout=button_layout)

# 选择颜色

choose_color = widgets.ToggleButtons(

options=['red', 'green', 'blue', 'yellow'],

button_style='success',

tooltips=['red', 'green', 'blue', 'yellow'])

# 取消追踪

follow_cancel = widgets.Button(description='follow_cancel', button_style='danger', layout=button_layout)

# 退出

exit_button = widgets.Button(description='Exit', button_style='danger', layout=button_layout)

# 图像控件

imgbox = widgets.Image(format='jpg', height=480, width=640, layout=widgets.Layout(align_self='auto'))

# 垂直布局

img_box = widgets.VBox([imgbox, choose_color], layout=widgets.Layout(align_self='auto'))

# 垂直布局

Slider_box = widgets.VBox([color_follow_btn, follow_cancel, exit_button],

layout=widgets.Layout(align_self='auto'))

# 水平布局

controls_box = widgets.HBox([img_box, Slider_box], layout=widgets.Layout(align_self='auto'))

def color_follow_Callback(value):

global model

model = 'color_follow'

def follow_cancel_Callback(value):

global model

model = 'General'

def exit_button_Callback(value):

global model

model = 'Exit'

color_follow_btn.on_click(color_follow_Callback)

follow_cancel.on_click(follow_cancel_Callback)

exit_button.on_click(exit_button_Callback)

def camera():

global HSV_learning, model

capture = cv.VideoCapture(0)

capture.set(3, 640)

capture.set(4, 480)

capture.set(5, 30)  # 设置帧率

while capture.isOpened():

try:

_, img = capture.read()

img = cv.resize(img, (640, 480))

if model == 'color_follow':

img = follow.follow_function(img, color_hsv[choose_color.value])

cv.putText(img, choose_color.value, (int(img.shape[0] / 2), 50),

cv.FONT_HERSHEY_SIMPLEX, 2, color[random.randint(0, 254)], 2)

if model == 'Exit':

cv.destroyAllWindows()

capture.release()

break

imgbox.value = cv.imencode('.jpg', img)[1].tobytes()

except KeyboardInterrupt:

capture.release()

devices = []

for i in range(5):

cap = cv.VideoCapture(i, cv.CAP_DSHOW)

if cap.isOpened():

devices.append(i)

cap.release()

print("可用设备索引:", devices)

# 显示控制面板

display(controls_box)

# 在后台线程中运行摄像头

camera_thread = threading.Thread(target=camera, daemon=True)

camera_thread.start()

del Arm

Color_follow

import cv2 as cv

import threading

import random

from time import sleep

import ipywidgets as widgets

from IPython.display import display

from Arm_Lib import Arm_Device

import Arm_Lib

Arm = Arm_Device()

joints_0 = [90, 135, 20, 25, 90, 30]

Arm.Arm_serial_servo_write6_array(joints_0, 1000)

class PositionalPID:

def __init__(self, P, I, D):

self.Kp = P # 比例系数

self.Ki = I # 积分系数

self.Kd = D # 微分系数

self.SystemOutput = 0.0  # 初始化系统的输出值

self.ResultValueBack = 0.0 # 初始化上次系统的输出值

self.PidOutput = 0.0 # 初始化PID控制器的输出值

self.PIDErrADD = 0.0 # 初始化PID误差累加值

self.ErrBack = 0.0 # 初始化上次的误差值

# 设置PID控制器参数

def SetStepSignal(self, StepSignal):

Err = StepSignal - self.SystemOutput  # 计算当前偏差

KpWork = self.Kp * Err  # 计算比例项

KiWork = self.Ki * self.PIDErrADD  # 计算积分项

KdWork = self.Kd * (Err - self.ErrBack) # 计算微分项

self.PidOutput = KpWork + KiWork + KdWork # 计算PID输出

self.PIDErrADD += Err # 更新PID误差累加值

self.ErrBack = Err # 更新上次的误差值

# 设置一阶惯性环节系统  其中InertiaTime为惯性时间常数, SampleTime为采样时间

def SetInertiaTime(self, InertiaTime, SampleTime):

# 根据惯性时间常数和PID输出计算当前系统输出

self.SystemOutput = (InertiaTime * self.ResultValueBack + \

SampleTime * self.PidOutput) / (SampleTime + InertiaTime)

# 更新上次系统的输出值

self.ResultValueBack = self.SystemOutput

class color_follow:

def __init__(self):

'''

初始化一些参数

'''

self.img = None

self.target_servox=90 # 机械臂1号舵机的初始角度

self.target_servoy=45

self.Arm = Arm_Lib.Arm_Device()

self.xservo_pid = PositionalPID(1.9, 0.3, 0.35)

self.yservo_pid = PositionalPID(1.9, 0.3, 0.35)

def follow_function(self, img, HSV_config):

(color_lower, color_upper) = HSV_config

self.img = cv.resize(img, (640, 480), )

self.img = cv.GaussianBlur(self.img, (5, 5), 0)

hsv = cv.cvtColor(self.img, cv.COLOR_BGR2HSV)

mask = cv.inRange(hsv, color_lower, color_upper)

mask = cv.erode(mask, None, iterations=2)

mask = cv.dilate(mask, None, iterations=2)

mask = cv.GaussianBlur(mask, (5, 5), 0)

cnts = cv.findContours(mask.copy(), cv.RETR_EXTERNAL, cv.CHAIN_APPROX_SIMPLE)[-2]

if len(cnts) > 0:

cnt = max(cnts, key=cv.contourArea)

(color_x, color_y), color_radius = cv.minEnclosingCircle(cnt)

if color_radius > 10:

# 将检测到的颜色用原形线圈标记出来

cv.circle(self.img, (int(color_x), int(color_y)), int(color_radius), (255, 0, 255), 2)

self.xservo_pid.SystemOutput = color_x # 圆心位置的x坐标作为PID的初始系统输出

self.xservo_pid.SetStepSignal(320) # 设置舵机在x轴方向上的目标位置为320，更新PID控制器参数

self.xservo_pid.SetInertiaTime(0.01, 0.1)# 设置PID的惯性时间

# 如果1号舵机的角度不大于180且圆心的x坐标大于320，或者1号舵机的角度值不小于0且圆心的x坐标小于320，则进行PID控制

if not (self.target_servox>=180 and color_x<=320 or self.target_servox<=0 and color_x>=320):

self.xservo_pid.SystemOutput = color_x

self.xservo_pid.SetStepSignal(320)

self.xservo_pid.SetInertiaTime(0.01, 0.1)

target_valuex = int(1500 + self.xservo_pid.SystemOutput)

self.target_servox = int((target_valuex - 500) / 10)

# 设置移动限制

if self.target_servox > 180:self.target_servox = 180

if self.target_servox < 0: self.target_servox = 0

#  如果1号舵机的角度不大于180且圆心的y坐标大于240，或者1号舵机的角度值不小于0且圆心的y坐标小于240，则进行PID控制

if not (self.target_servoy>=180 and color_y<=240 or self.target_servoy<=0 and color_y>=240):

# 输入Y轴方向参数PID控制输入

self.yservo_pid.SystemOutput = color_y

self.yservo_pid.SetStepSignal(240)

self.yservo_pid.SetInertiaTime(0.01, 0.1)

target_valuey = int(1500 + self.yservo_pid.SystemOutput)

self.target_servoy = int((target_valuey - 500) / 10) - 45

# 设置移动限制

if self.target_servoy > 360: self.target_servoy = 360

if self.target_servoy < 0: self.target_servoy = 0

# 调整机械臂各个舵机的旋转角度

joints_0 = [self.target_servox, 135, self.target_servoy / 2, self.target_servoy / 2, 90, 30]

self.Arm.Arm_serial_servo_write6_array(joints_0, 1000)

return self.img

follow = color_follow()

# 初始化模式

model = 'General'

# 初始化HSV_learning值

HSV_learning = ()

# 设置HSV值

color_hsv = {"red": ((0, 25, 90), (10, 255, 255)),

"green": ((53, 36, 40), (80, 255, 255)),

"blue": ((110, 80, 90), (120, 255, 255)),

"yellow": ((25, 20, 55), (50, 255, 255))}

# 设置随机颜色

color = [[random.randint(0, 255) for _ in range(3)] for _ in range(255)]

button_layout = widgets.Layout(width='200px', height='100px', align_self='center')

# 输出控件

output = widgets.Output()

# 颜色追踪

color_follow = widgets.Button(description='color_follow', button_style='success', layout=button_layout)

# 选择颜色

choose_color = widgets.ToggleButtons(options=['red', 'green', 'blue', 'yellow'], button_style='success',

tooltips=['Description of slow', 'Description of regular', 'Description of fast'])

# 取消追踪

follow_cancel = widgets.Button(description='follow_cancel', button_style='danger', layout=button_layout)

# 退出

exit_button = widgets.Button(description='Exit', button_style='danger', layout=button_layout)

# 图像控件

imgbox = widgets.Image(format='jpg', height=480, width=640, layout=widgets.Layout(align_self='auto'))

# 垂直布局

img_box = widgets.VBox([imgbox, choose_color], layout=widgets.Layout(align_self='auto'))

# 垂直布局

Slider_box = widgets.VBox([color_follow,follow_cancel,exit_button],

layout=widgets.Layout(align_self='auto'))

# 水平布局

controls_box = widgets.HBox([img_box, Slider_box], layout=widgets.Layout(align_self='auto'))

display(controls_box)

def color_follow_Callback(value):

global model

model = 'color_follow'

def follow_cancel_Callback(value):

global model

model = 'General'

def exit_button_Callback(value):

global model

model = 'Exit'

color_follow.on_click(color_follow_Callback)

follow_cancel.on_click(follow_cancel_Callback)

exit_button.on_click(exit_button_Callback)

def camera():

global HSV_learning,model

# 打开摄像头

capture = cv.VideoCapture(0)

capture.set(3, 640)

capture.set(4, 480)

capture.set(5, 30)  #设置帧率

# 当摄像头正常打开的情况下循环执行

while capture.isOpened():

try:

# 读取相机的每一帧

_, img = capture.read()

# 统一图像大小

img = cv.resize(img, (640, 480))

if model == 'color_follow':

img = follow.follow_function(img, color_hsv[choose_color.value])

# 添加文字

cv.putText(img, choose_color.value, (int(img.shape[0] / 2), 50), cv.FONT_HERSHEY_SIMPLEX, 2, color[random.randint(0, 254)], 2)

if model == 'Exit':

cv.destroyAllWindows()

capture.release()

break

imgbox.value = cv.imencode('.jpg', img)[1].tobytes()

except KeyboardInterrupt:capture.release()

display(controls_box,output)

threading.Thread(target=camera, ).start()

import cv2

# 列出通过 DirectShow 可检测的设备

devices = []

for i in range(5):

cap = cv2.VideoCapture(i, cv2.CAP_DSHOW)

if cap.isOpened():

devices.append(i)

cap.release()

print("可用设备索引:", devices)

del Arm

cap.release()

Last

#!/usr/bin/env python3

# coding=utf-8

import time

import cv2 as cv

import numpy as np

import matplotlib.pyplot as plt

from Arm_Lib import Arm_Device

Arm = Arm_Device()

joints_0 = [90, 135, 20, 25, 90, 30]

Arm.Arm_serial_servo_write6_array(joints_0, 1000)

time.sleep(1)

def arm_clamp_block(enable):

if enable == 0:  # 松开

Arm.Arm_serial_servo_write(6, 60, 400)

else:            # 夹紧

Arm.Arm_serial_servo_write(6, 135, 400)

time.sleep(0.5)

def arm_move(p, s_time=500):

for i in range(5):

id = i + 1

if id == 5:

time.sleep(0.1)

Arm.Arm_serial_servo_write(id, p[i], int(s_time * 1.2))

else:

Arm.Arm_serial_servo_write(id, p[i], s_time)

time.sleep(0.01)

time.sleep(s_time / 1000)

def arm_move_up():

Arm.Arm_serial_servo_write(2, 90, 1500)

Arm.Arm_serial_servo_write(3, 90, 1500)

Arm.Arm_serial_servo_write(4, 90, 1500)

time.sleep(0.1)

import cv2 as cv

import threading

import ipywidgets as widgets

from IPython.display import display

# 创建显示控件

camera_img = widgets.Image(format='jpg', width=640, height=480)

stop_btn = widgets.Button(description="停止画面", button_style='danger')

display(widgets.VBox([camera_img, stop_btn]))

# 全局变量控制线程

cap = cv.VideoCapture(0)

cap.set(3, 640)

cap.set(4, 480)

show_running = True

def show_camera_loop():

global show_running

while show_running:

ret, frame = cap.read()

if ret:

_, jpeg = cv.imencode('.jpg', frame)

camera_img.value = jpeg.tobytes()

def stop_show(b):

global show_running

show_running = False

cap.release()

print("画面已停止，摄像头释放")

stop_btn.on_click(stop_show)

# 启动线程

thread = threading.Thread(target=show_camera_loop, daemon=True)

thread.start()

print("摄像头实时画面已开启，可以继续执行下方单元格")

p_mould = [90, 130, 0, 0, 90]     # 取料点（模具上方）

p_top   = [90, 80, 50, 50, 270]   # 过渡高度

# 四个放置角落

p_Yellow = [64, 23, 68, 56, 270]

p_Red    = [114, 22, 66, 56, 270]

p_Green  = [134, 65, 20, 29, 270]

p_Blue   = [44, 65, 20, 28, 270]

# 四层积木的抓取位置（从高到低）

p_layer_4 = [90, 76, 40, 17, 270]

p_layer_3 = [90, 65, 44, 17, 270]

p_layer_2 = [90, 65, 22, 36, 270]

p_layer_1 = [90, 50, 37, 30, 270]

# 四层积木对应的识别站角度

IDENTIFY_POSITIONS = [

[86, 127, 3, 6, 90],    # layer_4（最高层）

[86, 119, 1, 8, 90],    # layer_3

[86, 111, 0, 11, 90],   # layer_2

[85, 79, 7, 25, 90]     # layer_1（最低层）

]

COLOR_THRESHOLDS = {

"red":    ((0, 50, 50),   (10, 255, 255)),

"green":  ((40, 50, 50),  (80, 255, 255)),

"blue":   ((100, 50, 50), (130, 255, 255)),

"yellow": ((20, 50, 50),  (40, 255, 255))

}

def detect_color_at_station(debug=True):

ret, frame = cap.read()

if not ret:

return None

blurred = cv.GaussianBlur(frame, (5, 5), 0)

hsv = cv.cvtColor(blurred, cv.COLOR_BGR2HSV)

h, w = hsv.shape[:2]

# 先用全图测试，确认积木能被抓到

# 如果你确定积木在中央，可以恢复 center_roi = hsv[...]

# 现在为了排查，直接用全图

roi = hsv  # 全图

max_area = 0

best_color = None

# 将面积阈值暂时降到 50，先跑通再说

AREA_THRESHOLD = 50

for color_name, (lower, upper) in COLOR_THRESHOLDS.items():

mask = cv.inRange(roi, np.array(lower), np.array(upper))

# 先不做形态学，看原始面积

area = cv.countNonZero(mask)

if debug:

print(f"{color_name}: {area} 像素")

if area > max_area and area > AREA_THRESHOLD:

max_area = area

best_color = color_name

if debug:

print(f"选中颜色: {best_color} (面积={max_area})")

return best_color

def sort_one_block(layer_pose, identify_pose, color_name=None):

"""

抓取指定层的积木，若未指定颜色则通过视觉识别，然后放到对应角落。

"""

# 1. 去取料点上方

arm_clamp_block(0)          # 确保夹爪张开

arm_move(p_mould, 1000)

time.sleep(0.5)

# 2. 移动到识别站

arm_move(identify_pose, 1500)

print("移动到识别位置")

time.sleep(0.5)

# 3. 识别颜色（如果未预知）

if color_name is None:

color_name = detect_color_at_station()

print(f"识别颜色: {color_name}")

# 4. 下降到模具表面

arm_move(p_top, 1000)

arm_move(layer_pose, 1000)

arm_clamp_block(1)          # 夹紧积木

time.sleep(0.3)

# 5. 抬升

arm_move(p_top, 1000)

# 6. 根据颜色选择放置位置

if color_name == 'yellow':

arm_move(p_Yellow, 1500)

elif color_name == 'red':

arm_move(p_Red, 1500)

elif color_name == 'green':

arm_move(p_Green, 1500)

elif color_name == 'blue':

arm_move(p_Blue, 1500)

else:

# 无法识别则放到默认位置（比如红色角落）

print("无法识别颜色，放到默认位置")

arm_move(p_Red, 1500)

# 7. 松开积木

arm_clamp_block(0)

time.sleep(0.3)

# 8. 抬升并回到安全位置

arm_move_up()

arm_move(p_mould, 1000)

if __name__ == '__main__':

print("开始分拣...")

# 依次处理四层积木（由高到低）

sort_one_block(p_layer_4,IDENTIFY_POSITIONS[0])

sort_one_block(p_layer_3,IDENTIFY_POSITIONS[1])

sort_one_block(p_layer_2,IDENTIFY_POSITIONS[2])

sort_one_block(p_layer_1,IDENTIFY_POSITIONS[3])

print("分拣完成")

del Arm

cap.release()

location

import cv2 as cv

import threading

import ipywidgets as widgets

from IPython.display import display

cap = cv.VideoCapture(0)

cap.set(3, 640)

cap.set(4, 480)

imgbox = widgets.Image(format='jpg', width=640, height=480)

display(imgbox)

def show_loop():

while True:

ret, frame = cap.read()

if ret:

_, jpeg = cv.imencode('.jpg', frame)

imgbox.value = jpeg.tobytes()

threading.Thread(target=show_loop, daemon=True).start()

print("摄像头画面已开启")

from Arm_Lib import Arm_Device

import time

Arm = Arm_Device()

Arm.Arm_Button_Mode(1)   # 开启自由模式，此时可以手动掰机械臂

print("自由模式已开启，请手动调整机械臂到识别位置")

Arm.Arm_Button_Mode(0)

print("识别站关节角度：")

ident_pos = []

for i in range(6):

angle = Arm.Arm_serial_servo_read(i+1)

ident_pos.append(angle)

print(f"舵机{i+1}: {angle}")

time.sleep(0.01)

print("\n复制下面这行到主程序：")

print(f"IDENTIFY_POS = {ident_pos}")

cap.release()

Porter

#!/usr/bin/env python3

#coding=utf-8

import time

from Arm_Lib import Arm_Device

Arm = Arm_Device()

time.sleep(.1)

def arm_clamp_block(enable):

if enable == 0:

Arm.Arm_serial_servo_write(6, 60, 400)

else:

Arm.Arm_serial_servo_write(6, 135, 400)

time.sleep(.5)

def arm_move(p, s_time = 500):

for i in range(5):

id = i + 1

if id == 5:

time.sleep(.1)

Arm.Arm_serial_servo_write(id, p[i], int(s_time*1.2))

else :

Arm.Arm_serial_servo_write(id, p[i], s_time)

time.sleep(.01)

time.sleep(s_time/1000)

def arm_move_up():

Arm.Arm_serial_servo_write(2, 90, 1500)

Arm.Arm_serial_servo_write(3, 90, 1500)

Arm.Arm_serial_servo_write(4, 90, 1500)

time.sleep(.1)

p_mould = [90, 130, 0, 0, 90]

p_top = [90, 80, 50, 50, 270]

p_Yellow = [64, 23, 68, 56, 270]

p_Red = [114, 22, 66, 56, 270]

p_Green = [134, 65, 20, 29, 270]

p_Blue = [44, 65, 20, 28, 270]

p_layer_4 = [90, 76, 40, 17, 270]

p_layer_3 = [90, 65, 44, 17, 270]

p_layer_2 = [90, 65, 30, 36, 270]

p_layer_1 = [90, 50, 37, 30, 270]

arm_clamp_block(0)

arm_move(p_mould, 1000)

time.sleep(1)

arm_move(p_top, 1000)

arm_move(p_layer_4, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_Yellow, 1000)

arm_clamp_block(0)

time.sleep(.1)

arm_move_up()

arm_move(p_mould, 1100)

arm_move(p_top, 1000)

arm_move(p_layer_3, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_Red, 1000)

arm_clamp_block(0)

time.sleep(.1)

arm_move_up()

arm_move(p_mould, 1100)

arm_move(p_top, 1000)

arm_move(p_layer_2, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_Green, 1000)

arm_clamp_block(0)

time.sleep(.1)

arm_move_up()

arm_move(p_mould, 1100)

arm_move(p_top, 1000)

arm_move(p_layer_1, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_Blue, 1000)

arm_clamp_block(0)

time.sleep(.1)

arm_move_up()

arm_move(p_mould, 1100)

del Arm

Pyramid

#!/usr/bin/env python3

#coding=utf-8

import time

from Arm_Lib import Arm_Device

Arm = Arm_Device()

time.sleep(.1)

def arm_clamp_block(enable):

if enable == 0:

Arm.Arm_serial_servo_write(6, 60, 400)

else:

Arm.Arm_serial_servo_write(6, 135, 400)

time.sleep(.5)

def arm_move(p, s_time = 500):

for i in range(5):

id = i + 1

if id == 5:

time.sleep(.1)

Arm.Arm_serial_servo_write(id, p[i], int(s_time*1.2))

else :

Arm.Arm_serial_servo_write(id, p[i], s_time)

time.sleep(.01)

time.sleep(s_time/1000)

p_mould = [90, 130, 0, 0, 90]

p_top = [90, 80, 50, 50, 270]

p_layer_4 = [90, 76, 40, 17, 270]

p_layer_3 = [90, 65, 44, 17, 270]

p_layer_2 = [90, 65, 30, 36, 270]

p_layer_1 = [90, 50, 37, 30, 270]

p_Yellow = [64, 22, 68, 56, 270]

p_Red = [114, 22, 66, 56, 270]

p_Green = [134, 60, 20, 29, 270]

p_Blue = [44, 65, 20, 28, 270]

arm_clamp_block(0)

arm_move(p_mould, 1000)

time.sleep(1)

# 夹取黄色区域的方块堆叠到中间最底层的位置。

arm_move(p_top, 1000)

arm_move(p_Yellow, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_layer_1, 1000)

arm_clamp_block(0)

time.sleep(.1)

arm_move(p_mould, 1100)

arm_move(p_top, 1000)

arm_move(p_Red, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_layer_2, 1000)

arm_clamp_block(0)

time.sleep(.1)

arm_move(p_mould, 1100)

arm_move(p_top, 1000)

arm_move(p_Green, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_layer_3, 1000)

arm_clamp_block(0)

time.sleep(.1)

arm_move(p_mould, 1100)

arm_move(p_top, 1000)

arm_move(p_Blue, 1000)

arm_clamp_block(1)

arm_move(p_top, 1000)

arm_move(p_layer_4, 1000)

arm_clamp_block(0)

time.sleep(.1)

arm_move(p_mould, 1100)

U_D_L_R

#!/usr/bin/env python3

#coding=utf-8

import time

from Arm_Lib import Arm_Device

Arm = Arm_Device()

time.sleep(.1)

# 同时控制六个舵机运动，逐渐变换角度。

def ctrl_all_servo(angle, s_time = 500):

Arm.Arm_serial_servo_write6(angle, 180-angle, angle, angle, angle, angle, s_time)

time.sleep(s_time/1000)

def main():

dir_state = 1

angle = 90

# 让舵机复位归中

Arm.Arm_serial_servo_write6(90, 90, 90, 90, 90, 90, 500)

time.sleep(1)

while True:

if dir_state == 1:

angle += 1

if angle >= 180:

dir_state = 0

else:

angle -= 1

if angle <= 0:

dir_state = 1

ctrl_all_servo(angle, 10)

time.sleep(10/1000)

try :

main()

except KeyboardInterrupt:

print(" Program closed! ")

Pass

del Arm

Untitled

#!/user/bin/env python3

#coding=utf-8

import time

from Arm_Lib import Arm_Device

Arm = Arm_Device()

time.sleep(.1)

def main():

for i in range(6):

aa = Arm.Arm_serial_servo_read(i+1)

print(aa)

time.sleep(.01)

time.sleep(.5)

print("EOF")

try:

main()

except KeyboardInterrupt:

print("Program closed")

pass

id = 4

angle = 90

Arm.Arm_serial_servo_write(id, angle, 500)

time.sleep(1)

aa = Arm.Arm_serial_servo_read(id)

print(aa)

time.sleep(.5)

Arm.Arm_Button_mode(1)